US Air Force Academy and Contractor Storm Water

Permitting Guidance

December 2023

(Replaces May 2022 version)

Project:  							`	Date

PURPOSE AND USE

This guidance document is intended to assist Contractors in complying with Environmental Protection Agency (EPA) 2022 Construction General Permit (CGP).  On January 18, 2022, EPA issued the CGP that took effect February 17, 2022, and will last for five years. Where EPA is the permitting authority (see below), construction stormwater discharges are almost all permitted under the Construction General Permit (CGP). The SWPPP is not required to follow the format of the checklist, though it is strongly recommended for ease of SWPPP preparation and CE Water Quality Manager review and approval.

STORM WATER PERMITTING REQUIREMENTS

Construction General Permit

Proposed USAFA construction projects consisting of one or more acres of land disturbance are regulated by EPA’s CGP.  The CGP authorizes storm water discharges from these construction activities within limitation.  The goal of the CGP is to reduce or eliminate storm water pollution from construction activities by requiring the proponent (Contractor) plan and implement appropriate pollution control practices to protect water quality.  The CGP is administered by the EPA for federal lands.  The State of Colorado currently has no jurisdiction regarding storm water compliance on federal land.

Land disturbance includes clearing, grubbing, grading, excavation, stockpiling, maintenance yards, parking areas, and other activities creating land disturbance where vegetation is removed, or significantly impacted, creating erosion prone areas including vehicle wheel disturbance.  Routine maintenance and repair of existing gravel and asphalt roads and parking lots is not included in the acreage calculation.  New road and parking lot construction is considered land disturbance and is included in the acreage threshold.

All proposed projects of one or more acres are required to fulfill CGP requirements.  CGP waivers for Certain Small Construction Activities (CGP Appendix C) do not apply and will not be accepted at USAFA.

The CGP and SWPPP are considered binding compliance documents.  Failure to comply with CGP and SWPPP may result in the EPA issuing a Notice of Violation against the Contractor with potentially significant monetary penalty.

In addition to CGP compliance requirements, the Academy maintains an individual Municipal Separate Storm Sewer System (MS4) Permit - COR042007, which includes a construction BMP overlay.  The MS4 Permit requires BMPs be identified in the Construction Site Storm Water Runoff Control BMP with measurable goals.  Though the Construction General Permit’s SWPPP compliance requirements encompass construction BMPs, these BMPs are reiterated in the MS4 storm water management program including the design and installation of post-construction storm water BMPs as required.  A violation of the CGP can be considered a violation of the MS4 Permit.

Notice of Intent(s)

The EPA requires the Contractor to electronically prepare and submit your Notice of Intent (NOI) for coverage under the 2022 CGP, unless you received a waiver from your EPA Regional Office, through the eReporting Tool (NeT).   The Contractor is defined as being the day-to-day operator and the one responsible for SWPPP and CGP compliance.  USAFA, as the landowner, has CGP compliance oversight, and having control over Plans and Specifications.  Prior to filing the NOI, the Contactor is required to prepare and submit a SWPPP to the USAFA Environmental Program Manager for approval.  The storm water permitting process is required to be completed prior to construction commencement.  The CE Environmental Program Manager also provides the Contractor with other terms and conditions based upon USAFA’s Erosion Control, Revegetation, and Tree Care Standard (Section 01351, rev. 2019) and USAFA’s Environmental Standards (Section 01350, rev. 2021).  The following EPA website will provide you access to the CGP and related NOI links: Submitting a Notice of Intent (NOI), Notice of Termination (NOT), or Low Erosivity Waiver (LEW) under the Construction General Permit.

Notice of Termination

The Contractors electronic Notice of Termination (NOT) through NeT filing is required to be approved by the CE Environmental Program Manager prior to filing.  For NOT consideration, contact your CE Construction Representative in writing requesting termination.  The CE Construction Representative will notify the CE Environmental Program Manager who will perform the termination assessment.  Typically, for adequate vegetation or stabilization to occur, the project warranty period should be near expiration for termination consideration.  In some situations, the Academy may authorize earlier termination depending on site conditions and compliance with the CGP.  However, the Contractor is required to comply with the Overarching Environmental Specifications Revegetation Specification including required topsoil replacement and other specific revegetation specifications.  If the Contractor does not implement or comply with the Revegetation Specifications, the Academy will not consider authorizing the NOT.

Completion of the Final Contract Punch List/Final Inspection performed between the Contracting Officer and the CE Construction Representative, does not relieve the Contractor of continued compliance under the SWPPP and General Permit.  Expiration of the contract warranty period does not relieve the Contractor of continued compliance under the SWPPP and CGP.

Project Ultimately Exceeds One Acre

If initially the project is estimated to disturb less than one-acre, and the disturbance is actually one or more acres during construction, the Contractor is to immediately notify the CE Environmental Program Manager and initiate compliance with the CGP and complete the procedures identified in this document.  Any proposed project estimating total disturbed acreage of between 0.8 and 0.99 acres are encouraged to obtain storm water permit coverage.  Project delay due to a contractor not preparing an approved SWPPP and NOI filing will be the contractor’s responsibility.

Projects Less Than One Acre

If proposed land disturbance is less than one-acre, the Contractor is required to implement storm water Best Management Practices (BMPs) without preparing a SWPPP or filing an NOI.  The USAFA may require the Contractor prepare a Storm Water Management Plan, prior to construction, if construction activities are less than one-acre but will occur in an environmentally sensitive or highly erosion prone areas.  These projects are required to comply with the USAFA Revegetation Specification which provides compliance with USAFA’s Municipal Storm Water General Permit.

CONTRACTOR AND USAFA ROLES

Role of the Contractor

The Contractor shall be familiar with CGP compliance requirements and is responsible for complying with SWPPP and CGP terms.  The Contractor is required to prepare a signed SWPPP and submit the SWPPP to the USAFA Contracting Office.  Once the SWPPP has been approved by CE Environmental Program Manager, the Contractor will provide a signed copy of the final SWPPP to the Contracting Office and/or the CE Environmental Program Manager.  Once the SWPPP is approved, the Contactor can file their NOI electronically.  The EPA NeT approval process is, at a minimum, a 14-day process (waiting) before it is activated.  Land disturbing construction activities are not authorized until the NOI is activated.

The Contractor performs and documents inspections pursuant to CGP requirements and maintains the SWPPP (living project document) for EPA and USAFA inspection.  Upon NOT, the Contractor is required to archive the SWPPP for three-years in the event of an EPA inspection.  CE will archive the original approved SWPPP with related CE inspection forms.

Role of USAFA Construction Representative and USAFA Contracting Office

The Construction Representative is responsible for coordinating the SWPPP process with the Contracting Office.  The SWPPP is given to the Contracting Office by the Contractor as a submittal requirement (Submittal Register Category A).  The Contracting Office submits the SWPPP to the CE Construction Representative.  The SWPPP requires government approval, and the Construction Representative submits the SWPPP to the CE Environmental Program Manager for review and written approval, or disapproval with comment.  The Contracting Office, or the Construction Representative, does not review the SWPPP for CGP compliance.

The Construction Representative performs and documents weekly inspections to demonstrate compliance with the CGP.  These reports are submitted to the CE Environmental Program Manager for review.  In the event the inspection report identifies any deficiencies, or potential non-compliance, a copy of the inspection report is sent to the project Contracting Officer.  The Contracting Officer solicits a written response from the Contractor regarding corrective actions taken and sends the corrective action response to the Construction Representative and Environmental Program Manager.  Failure for a Contractor not completing corrective action within a timely manner will subject the Contractor to potential project delay at their cost.

Role of CE Environmental Program Manager

The CE Environmental Program Manager represents the USAFA regarding compliance with the Clean Water Act.  As an officer under the USAFA MS4, the Environmental Program Manager has direct Clean Water Act compliance oversight.

The CE Environmental Program Manager provides guidance to Contractors regarding storm water permitting.  The CE Environmental Program Manager is available to answer questions regarding SWPPP preparation, NOI and NOT electronic filing, and CGP compliance requirements.  The Environmental Program Manager approves, or disapproves, the Contractors SWPPP.  Once the SWPPP has been approved by the Environmental Program Manager, the approval is noted which is given to the Construction Representative who submits the form to the Contracting Office.

The Water Quality Manager does not maintain the Contractor’s SWPPP for compliance demonstration, but retains a copy of the approved SWPPP, CE inspection reports and NOI termination correspondence.

Additional NeT information for Contractor filing is provided by the Environmental Program Manager and includes the Endangered Species Act category selection (A through F) and Total Maximum Daily Load identification.  The Contractor may proceed with construction once the Contractor’s NOI is activated.  The USAFA does not wait for their NOI to be activated by EPA since the Contractor’s NOI has been activated.

2022 STORM WATER POLLUTION PREVENTION PLAN

(SWPPP) CHECKLIST

This checklist is intended to reflect EPA requirements for a compliant SWPPP.  The checklist serves as a guidance document only.  The ultimate responsibility for preparing the SWPPP and completing the electronic Notice of Intent (NOI) process rests with the Contractor.  The use of this checklist does not guarantee compliance with Construction General Permit COR12000F (CGP).

The “Permit Citation” column indicates where the particular requirement can be found in the Construction CGP.  Use the “Location” column to note where the requirement can be found in your SWPPP.  Use the “check” column to indicate the SWPPP adequately addresses the corresponding requirements.  Submit completed checklist as part of the SWPPP for CE Environmental Program Manager review.  The Description briefly identifies the CGP SWPPP requirement while the italicized notation identifies procedural or responsibility clarification.

A full description of the required contents of the SWPPP, as well as additional permit requirements, may be found in the CGP effective February 17, 2022.  This permit is available on-line at: https://www.epa.gov/npdes/2022-construction-general-permit-cgp#2022cgp or by contacting the CE Environmental Program Manager.

| 2022 General Permit for Storm Water Discharges from Construction Activities COR12000F – SWPPP Checklist |  |  |  |
| --- | --- | --- | --- |
| 2022 CGP Citation | Description | Check | Location in SWPPP |
| Miscellaneous SWPPP Contents |  |  |  |
| General | On SWPPP cover page, note General Permit COR12000F. Include completed checklist in SWPPP for USAFA submittal and review. |  |  |
| General | Upon SWPPP approval, place completed USAFA SWPPP checklist in SWPPP. |  |  |
| General | All CGP SWPPP requirements must be identified in the Contractor’s SWPPP. If a CGP SWPPP requirement does not apply, the SWPPP is required to identify the requirement and briefly explain why the requirement does not apply. Please use EPA’s 2022 SWPPP template found at: https://www.epa.gov/npdes/construction-general-permit-resources-tools-and-templates#swppp |  |  |
| Appendix I, Part 1.11.4. | Certify SWPPP in accordance to General Permit. Do not modify certification language. |  |  |
| (2022 CGP)- Part 1.5 | The Contractor must post a sign or other notice of the permit coverage at a safe, publicly accessible location in close proximity to the construction site. The notice must be located so that it is visible from the public road that is nearest to the active part of the construction site, and it must use a font large enough to be readily viewed from a public right-of-way. At a minimum, the notice must include: If - The NPDES ID (i.e., permit tracking number assigned to the NOI); - A contact name and phone number for obtaining additional construction site information; - The Uniform Resource Locator (URL) for the SWPPP (if available), or the following statement: “If you would like to obtain a copy of the Stormwater Pollution Prevention Plan (SWPPP) for this site, contact the EPA Regional Office at [include the appropriate CGP Regional Office contact information found at https://www.epa.gov/npdes/contact-us-stormwater#regional];” and - The following statement “If you observe indicators of stormwater pollutants in the discharge or in the receiving waterbody, contact the EPA through the following website: https://www.epa.gov/enforcement/report-environmental-violations.” |  |  |
| (2022 CGP)- Appendix I, Part I.15 | The Contractor is required to obtain CE Environmental Program Manager approval of final stabilization prior to filing NOT for permit termination. Copies of the SWPPP and all documentation required by this permit, including records of all data used to complete the NOI to be covered by this permit, must be retained for at least three years from the date that permit coverage expires or is terminated. This period may be extended by request of EPA at any time. USAFA is also required to retain the original approved SWPPP and CE inspections for 3 years. |  |  |
| (2022 CGP)- Part 7.2.11 | Post-Authorization Additions to the SWPPP. Are the following items provided in the SWPPP? - A copy of the NOI submitted to EPA along with any correspondence exchanged between Permittee and EPA related to coverage under this permit; - A copy of the acknowledgment letter received from NeT assigning your NPDES ID (i.e., permit tracking number); - A copy of this permit (an electronic copy easily available to the stormwater team is also acceptable). After CE Environmental Program Manager approval, include signed and certified copy of the activated NOI in the SWPPP. |  |  |
| (2022 CGP)- Part 7.4.2 | Is there an SWPPP amendment log showing all the dates of the SWPPP modifications? This log must include the name and person authorizing the date of each change and a brief summary of all changes. |  |  |
| General | Reference and Implement current USAFA Revegetation Specifications for final stabilization practices. Transition from construction BMPs to post-construction BMPs should be considered and identified in the SWPPP. |  |  |
| General | Provide a description of final stabilization practices and BMPs. |  |  |
| SWPPP Template – Section 1: Contact Information/ Responsibilities Designates operators, subcontractors, and the Stormwater team Provides a 24-hour emergency contact |  |  |  |
| Para 1.1 – Operators and Subcontractors |  |  |  |
| 2022 CGP SWPPP template (1.1) CGP App 7.2.1 and I.11 | Identify the operator(s) who will be engaged in construction activities at the site. Indicate respective responsibilities, where appropriate. Also include the 24-hour emergency contact. List subcontractors expected to work on-site. Notify subcontractors of stormwater requirements applicable to their work. |  |  |
| Para 1.2 – Stormwater Team |  |  |  |
| 2022 CGP SWPPP template (1.2) (2022 CGP) - Section 7.2.2- Stormwater Team and CGP Appendix I | Identify the individuals (by name or position) that are part of the project’s stormwater team, their individual responsibilities, and which members are responsible for inspections. At a minimum the stormwater team is comprised of individuals who are responsible for overseeing the development of the SWPPP, any later modifications to it, and for compliance with the permit requirements (i.e., installing and maintaining stormwater controls, conducting site inspections, and taking corrective actions where required). Each member of the stormwater team (including CE Environmental Program Manager) must have ready access to either an electronic or paper copy of applicable portions of the 2022 CGP and the SWPPP. |  |  |
| SWPPP Template – Section 2: Site Evaluation, Assessment, and Planning Describes the construction sequence of the project How project will be carried out (e.g. scope, phasing, schedule) |  |  |  |
| Para 2.1 – Project/Site Information |  |  |  |
| 2022 CGP SWPPP template (2.1) (2022 CGP) – Part 7.2.4 | Provide project name and address, location (lat/long), method used to determine location and additional project information (i.e. is project in Indian country lands, in response to public emergency or a federal operator?) |  |  |
| Para 2.2 – Discharge Information |  |  |  |
| 2022 CGP SWPPP template (2.2) (2022 CGP)- Part 7.2.4.c MS4 Program | Discharge information- List the name of surface waters that receive discharges from your construction site or from the MS4. Are any of the waters listed “impaired” by state or tribe within one mile downstream of the sites discharge point? Please provide specific information on pollutants causing impairment. Monument Creek is currently impaired for E. coli. |  |  |
| 2022 CGP SWPPP template (2.2) (2022 CGP)- Part 2.2.1, Part 7.2.4.d Appendix D MS4 Program | Are any surface waters located within 50 feet of project? Have the buffer requirements been satisfied per section 2.2.1? Are there exceptions (i.e. no discharge of stormwater to surface water) or exemptions (404 permit, water dependent structure or water access area) and are they identified in SWPPP? Please place copy of 404 permits or other documentation that fulfils buffer requirements. |  |  |
| Para 2.3 – Nature of Construction Activity |  |  |  |
| 2022 CGP SWPPP template (2.3.1) (2022 CGP)- Part 7.2.3 | Describe the general nature/function of the construction project. |  |  |
| 2022 CGP SWPPP template (2.3.2) (2022 CGP)- Part 7.2.3.b/c/e | Indicate the total site area (in acres) that is expected to be disturbed (including off-site borrow areas, fill areas, stockpiles and maintenance yard). Note on project map by boundary inclusive project area. Soil disturbance and vegetation impacts resulting from the construction activity will be included in the disturbed acreage calculation. The Contractor is responsible for final stabilization of these areas. |  |  |
| 2022 CGP SWPPP template (2.3.3) (2022 CGP)- Part 7.2.3.d | Are there construction support activities described? |  |  |
| 2022 CGP SWPPP template (2.3.4) (2022 CGP)- Part 7.2.3.h | Are the business hours and work days included? |  |  |
| Part 2.4 - Sequence and Estimated Dates of Construction Activities |  |  |  |
| 2022 CGP SWPPP template (2.4) (2022 CGP)- Part 7.2.3.f | Describe the intended sequence of major construction activities that disturb soils at the construction site. Provide estimated sequence time frames (dates) of major construction activities. |  |  |
| Part 2.5 – Allowable Non-Stormwater Discharges |  |  |  |
| 2022 CGP SWPPP template (2.5) (2022 CGP)- Part 7.2.5 and 1.2.2 | Are all allowable sources of non-stormwater discharges identified? (ex. Construction dewatering, hydrostatic testing) Note: these may require a separate process water discharge permit. |  |  |
| Part 2.6 – Site Maps |  |  |  |
| 2022 CGP SWPPP template (2.6) (2022 CGP)-Part 7.2.4 | Is there a site map documenting equipment staging areas, material supply, track out pads, total soil disturbance area, drainage patterns, outfalls, etc.? Refer to maps in Appendix A. |  |  |
| SWPPP Template - Section 3: Documentation of Compliance with other Federal Requirements NEPA Review, Endangered Species, Historical Preservation, and Safe Drinking Water Act |  |  |  |
| Para 3.1 – Endangered Species Protection |  |  |  |
| 2022 CGP SWPPP template (3.1, Appendix K) (2022 CGP)-Part 7.2.9.a | Has documentation been included to support the determination of eligibility criterion (A-F) that has been met with the respect to the protection of endangered species? Include any relevant documentation from U.S. Fish and Wildlife Service or equivalent USAFA correspondence in the SWPPP. |  |  |
| Para 3.2 – Historic Preservation |  |  |  |
| 2022 CGP SWPPP template (3.2 and Appendix K) (2022 CGP) – 7.2.9.b | Has documentation been included that support the determination of whether installation of subsurface earth-disturbing stormwater controls will have on effect on historic properties? Include any relevant documentation or equivalent USAFA correspondence in the SWPPP. |  |  |
| Para 3.3 – Safe Drinking Water Act – Underground Injection Control Requirements |  |  |  |
| 2022 CGP SWPPP template (3.3) (2022 CGP)-Part 7.2.9.c | Are any of the following controls (Infiltration trenches, subsurface vaults, devices used to capture and infiltrate stormwater, drywells, seepage pits, improved sinkholes) going to be installed? Has there been coordination with applicable state or EPA regional office for implementing the requirements for Underground injection wells? |  |  |
| Record of Environmental Consideration/NEPA Review |  |  |  |
| 2022 CGP SWPPP template (Appendix K) (2022 CGP)-Part 7.2.9.a/b | Is the Record of Environmental Consideration (REC) included? |  |  |
| SWPPP Template- Section 4: Erosion and Sediment Controls Provides detailed erosion and sediment control plans and specs; outlines temporary and final stabilization plans |  |  |  |
| Para 4.1 – Natural Buffers or Equivalent Sediment Controls |  |  |  |
| 2022 CGP SWPPP template (4.1.1) (2022 CGP)-Part 2.2.1 and 7.2.6.b | Were surface waters within 50 feet from the project considered? Encourage to utilize existing vegetation as buffer. Preserve vegetation where possible and reference current USAFA Revegetation Specifications. |  |  |
| 2022 CGP SWPPP template (4.1.1) (2022 CGP)-Part 2.2.1 and 7.2.6.b | If surface waters are within 50 feet, were alternatives considered? |  |  |
| 2022 CGP SWPPP template (4.1.2) (2022 CGP)-Part 2.2.1 and 7.2.6.b | Are buffer exceptions considered? |  |  |
| Para 4.2 – Sediment Controls |  |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does the SWPPP include a description and specifications for sediment and perimeter controls? |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does SWPPP include installation dates for sediment controls? |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does SWPPP include maintenance requirements for sediment and perimeter controls? |  |  |
| Para 4.3 – Vehicle Tracking Control BMPs |  |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does the SWPPP include a description and specifications for vehicle tracking control (VTC)? |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does the SWPPP include installation dates for VTC? |  |  |
| (2022 CGP)- 7.2.6.b.ii | Does the SWPPP include maintenance requirements for VTC? |  |  |
| Para 4.4 – Stockpile Sediment and Soil |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for stockpile controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for stockpile controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for stockpile controls? |  |  |
| Para 4.5 – Minimize Dust |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for dust controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for dust controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for dust controls? |  |  |
| Para 4.6 – Minimize the Disturbance on Steep Slopes |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for steep slope controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for steep slope controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for steep slope controls? |  |  |
| Para 4.7 – Topsoil |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for topsoil controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for topsoil controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for topsoil controls? |  |  |
| Para 4.8 – Soil Compaction |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for soil compaction controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for soil compaction controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for soil compaction controls? |  |  |
| Para 4.9 – Storm Drain Inlet Protection |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for storm drain inlet protection? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for storm drain inlet protection? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for storm drain inlet protection? |  |  |
| Para 4.10 – Constructed Stormwater Conveyance Channels |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for stormwater channels? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for stormwater channels? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for stormwater channels? |  |  |
| Para 4.11 – Sediment Basins |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for sediment basins? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for sediment basins? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for sediment basins? |  |  |
| Para 4.12 – Chemical Treatment |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for chemical treatment? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for chemical treatment? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for chemical treatment? |  |  |
| Para 4.13 – Dewatering Practices |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for dewatering? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for dewatering? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for dewatering? |  |  |
| Para 4.14 – Other Stormwater Controls |  |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include a description and specifications for other controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include installation dates for other controls? |  |  |
| (2022 CGP)- 7.2.6.a | Does the SWPPP include maintenance requirements for other controls? |  |  |
| Para 4.15 – Temporary Stabilization Measures (For Each Measure) |  |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include a description and specifications for temporary stabilization measures? |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include installation dates for temporary stabilization measures? |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include maintenance requirements for temporary stabilization measures? |  |  |
| Permanent Stabilization (For Each Measure) |  |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include a description and specifications for permanent stabilization measures? |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include installation dates for permanent stabilization measures? |  |  |
| (2022 CGP)- 7.2.6.b.vii | Does the SWPPP include maintenance requirements for permanent stabilization measures? |  |  |
| Para 4.16 – Final Stabilization |  |  |  |
| (2022 CGP) – 2.2.14.b | Does the SWPPP state the USAFA Stormwater Program is required to conduct a final inspection? 70% vegetative cover (or equivalent) is required before a NOT will be issued? |  |  |
| Para 4.17 - Documentation |  |  |  |
| MS4 Program | Does the SWPPP address the procedures for documenting temporary and permanent stabilization measures? |  |  |
| Low Impact Design (LID) |  |  |  |
| MS4 Program | Does the SWPPP identify LID stormwater management measures to address stormwater runoff once the construction is complete? |  |  |
| SWPPP Template- Section 5: Pollution Prevention Standards Identifies activities and potential pollutants |  |  |  |
| 2022 CGP SWPPP template (5.1) (2022 CGP)- Part 7.2.6.vii | Does the SWPPP identify potential pollution generating activities and pollutants which could be discharged from the site? (e.g. paving operations; paint, stucco, form release oils curing compounds, masonry, and concrete cuttings/washouts; solid waste disposal and storage, etc.). |  |  |
| 2022 CGP SWPPP template (5.2) (2022 CGP)- Part 7.2.11 | Does the SWPPP have a spill plan? This would include procedures that will prevent and respond to leaks, spills, and other releases. The Academy requires that you develop a Spill Prevention and Response Plan (SPRP) and proposed use of hazardous materials requires USAFA approval (Form 3952). |  |  |
| 2022 CGP SWPPP template (5.3) (2022 CGP)- Part 2.3.1 | Are there pollution prevention measures specified for fueling and maintenance of equipment and vehicles? Have practices been identified that will eliminate the discharge of spilled or leaked chemicals? |  |  |
| 2022 CGP SWPPP template (5.4) (2022 CGP)- Part 2.3.2 | Are there pollution prevention measures specified for equipment and vehicle washing? Have practices been identified that will eliminate the discharge of pollutants from equipment and vehicle washing, wheel wash water, and other types of washing? Are there preventative measures to eliminate the discharge of soaps, detergents, or solvents? |  |  |
| 2022 CGP SWPPP template (5.5) See Para 5.5.1 to 5.5.6 (2022 CGP)- Part 2.3.3 | Are there pollution prevention measures specified for storage, handling, and disposal of building products, materials, and waste? Have practices been identified that will eliminate the discharge of these wastes? |  |  |
| 2022 CGP SWPPP template (5.5.1) (2022 CGP)- Part 2.3.a | Building Products |  |  |
| 2022 CGP SWPPP template (5.5.2) (2022 CGP)- Part 2.3.3.b | Pesticides, Herbicides, Insecticides, Fertilizers, and Landscape Materials |  |  |
| 2022 CGP SWPPP template (5.5.3) (2022 CGP)- Part 2.3.3.c | Fuels. Oils, Fluids, and Other Chemicals |  |  |
| 2022 CGP SWPPP template (5.5.4) (2022 CGP)- Part 2.3.3.d | Hazardous or Toxic Waste |  |  |
| 2022 CGP SWPPP template (5.5.5) (2022 CGP)- Part 2.3.3.e | Construction and Domestic Waste |  |  |
| 2022 CGP SWPPP template (5.5.6) (2022 CGP)- Part 2.3.3.f | Sanitary Waste |  |  |
| 2022 CGP SWPPP template (5.6) (2022 CGP)- Part 2.3.4 | Are pollution prevention measures specified for washing applicators and containers for paint, concrete, and other materials? (e.g. lined concrete wash out) |  |  |
| 2022 CGP SWPPP template (5.7) (2022 CGP)- Part 2.3.5 | Are pollution prevention measures specified for fertilizers? |  |  |
| 2022 CGP SWPPP template (5.8) (2022 CGP)- Part 2.3.6 | Are other pollution prevention measures specified? |  |  |
| SWPPP Template- Section 6: Inspections, Corrective Actions, and Modifications Outlines inspection plan (including daily precipitation record) |  |  |  |
| 2022 CGP SWPPP template (6.1.1) (2022 CGP)-Part 4.1 | Are inspections being performed by a qualified person and are their qualifications in the SWPPP? |  |  |
| 2022 CGP SWPPP template (6.1.1) MS4 Program | Does the SWPPP include the following statement? “The USAFA Stormwater Program will conduct quarterly inspections to check compliance with the CGP. The Environmental Program Manager will contact the project proponent to schedule site visits, check the SWPPP updates, and check that routine inspections are performed.” |  |  |
| 2022 CGP SWPPP template (6.1.2) (2022 CGP)-Part 4.2 and 7.2.7 | Is the frequency of, and procedures for, inspections noted in the SWPPP? |  |  |
| 2022 CGP SWPPP template (6.1.2) (2022 CGP)-Part 4.4 | Does the Plan address a reduction in the inspection schedule for stabilized areas and halts in construction activities? |  |  |
| 2022 CGP SWPPP template (6.1.3) (2022 CGP)-Part 4.5 | Are the areas that must be inspected included? |  |  |
| 2022 CGP SWPPP template (6.1.4) (2022 CGP)-Part 4.6 | Are inspection requirements included? |  |  |
| 2022 CGP SWPPP template (6.1.5) See Appendix D (2022 CGP)-Part 4.7 | Is the inspection form adequate? |  |  |
| 2022 CGP SWPPP template (6.1.5) (2022 CGP)-Part 4.7 | Is the frequency of, and procedures for, inspections noted in the SWPPP? |  |  |
| Para 6.2 – Corrective Action |  |  |  |
| 2022 CGP SWPPP template (6.2.1) (2022 CGP)-Part 5.1 | Are the conditions triggering corrective actions specified? |  |  |
| 2022 CGP SWPPP template (6.2.2) (2022 CGP)-Part 5.2 | Does the Plan address the corrective action deadlines? |  |  |
| 2022 CGP SWPPP template (6.2.3) | Does the SWPPP address procedures for documenting corrective actions? |  |  |
| 2022 CGP SWPPP template (6.2.4) See Appendix E (2022 CGP)-Part 5.4 | Does the SWPPP have a copy of the corrective actions report form that will be used for the project? |  |  |
| Para 6.3 – Modification |  |  |  |
| (2022 CGP)-Part 7.4 | Is there a process for modifying the SWPPP? |  |  |
| (2022 CGP)-Part 7.4.2 See Appendix F | Is there an appendix for post-authorization additions to the SWPPP (i.e. change log)? |  |  |
| 2022 CGP SWPPP template (6.2.4) See Appendix E (2022 CGP)-Part 5.4 | Does the SWPPP have a copy of the corrective actions report form that will be used for the project? |  |  |
| Para 6.4 – Records Maintenance |  |  |  |
| (2022 CGP)-Part 4.7.4 and 5.4.5 | Does the SWPPP state the requirement to retain all records for 3 years after completion of project? |  |  |
| SWPPP Template- Section 7: Training Includes credentials of qualified individuals who are managing the SWPPP |  |  |  |
| (2022 CGP)-Part 6 and 7.2.8 See Appendix L | Have the individuals that are responsible for SWPPP implementation, BMP installation and maintenance, inspections, pollution prevention measures and corrective actions reports been trained and is there documentation of the training? Has this been initiated prior to earth-disturbing activities? |  |  |
| (2022 CGP)-Part 6.2 | Does the Plan describe how subcontractors and new personnel will be trained on the SWPPP? |  |  |
| (2022 CGP)-Part 6.2 | Does the SWPPP address a routine training plan for employees and subcontractors? (Recommend monthly tailgate training) |  |  |
| (2022 CGP)-Part 6.2 See Appendix G | Does the SWPPP have a subcontractor agreement form? |  |  |
| SWPPP Template- Section 8: Certification and Notification Signed certification from the contractor (Legal responsibility and liability) |  |  |  |
| CGP Application I.11 | Did all “operators” sign the SWPPP? |  |  |
| CGP Application I.11 | Did all “operators” sign the SWPPP? |  |  |
| (2022 CGP) Part 7.2.1.5 and Application I.11.4 | Is there a signed and dated certification statement that meets the requirements of Appendix I, Part 1.11? |  |  |
| Appendices |  |  |  |
| Appendix A | Project Maps - Site Map - BMP Plan (by phase) - Stabilization Plan - Final Grading Plan |  |  |
| Appendix B | Copy of the 2022 CGP |  |  |
| Appendix C | NOI Application and EPA authorization (place holder) |  |  |
| Appendix D | Inspection Form |  |  |
| Appendix E | Corrective Action Log and Form |  |  |
| Appendix F | SWPPP Amendment Log |  |  |
| Appendix G | Subcontractor Certification and Agreements |  |  |
| Appendix H | Grading and Stabilization Log |  |  |
| Appendix I | Training Log |  |  |
| Appendix J | Delegation of Authority |  |  |
| Appendix K | Record of Environmental Consideration (NEPA) |  |  |
| Appendix M | BMP Details and Specifications |  |  |
| Appendix N | Seeding Specifications |  |  |
| Appendix O | USAFA Dewatering Requirements and Guidance |  |  |
| Appendix P | USAFA Spill Plan and Action Guidance |  |  |

The statements in this document are intended solely as guidance to aid Contractors in complying with Construction General Permit COR12000F requirements.  Compliance with this document does not preclude compliance with all Construction General Permit terms, or deviation from the approved plans and specifications.