Appendix A

RFP-25-106

El Paso County Public Water System Operator in Responsible Charge Services

Task or Activity Description, Authorized Person(s) or Position(s), Operational Limits, and Response provided.

| Task or Activity Description | Authorized Person(s) or Position(s) | Operational Limits and Response |
| --- | --- | --- |
| Entry point and distribution system sampling | ORC. El Paso Parks employees. | El Paso Parks employees will only take CL2 residual samples. If CL2 residual is measured below 0.2mg/L or above 4.0mg/L at any time, ORC shall be notified immediately. ORC will handle all other samplings. |
| Weekly reads | ORC. El Paso Parks employees. | Weekly CL2 and meter reads taken. Any changes noted. |
| Flow changes. | ORC. | CL2 adjustment. Pressure and flow adjustment. |
| Chemical top offs | ORC. El Paso Parks employees. | Addition of bleach to chemical day-tank. |
| Ion Exchange/Water Softener | ORC. | Adjustment of water softener backwashes/regeneration units. |
| System(s) cleaning | ORC. El Paso Parks employees. | Basic cleaning and organization of systems infrastructure. |
| Check storage tanks, infrastructure, and distribution system(s) | ORC. El Paso Parks employees. | Visual checks for damage, leaks, security integrity, changes, or inconsistencies with system(s). |
| Nitrate and Total Coliform sampling | ORC. | Bac-t every quarter. Nitrite every year. |
| Repairs, unscheduled maintenance, and system reconfigurations | ORC. | Will assess and perform work. |
| Seasonal startup procedures (Homestead) | ORC. | Conduct startup procedures and certification submission. |